|  | 72095 VIS#023939 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Part Number | Thermal Resistance °C/W at 3in length | Width in | Height in | Surface Area in/n | Weight lb/ft |
|  |  | 72095 | 1.57 | 2.60 | 2.19 | 44.6 | 2.30 |

| Thermal Curves based on 3.000 in length Top of Form Bottom of Form |  |
| --- | --- |

Natural Convection

|  | Heat Sink Temperature Rise Above Ambient |  |  | Heat Sink Thermal Resistance |
| --- | --- | --- | --- | --- |
|  | 25.0. 18.8. 12.5. 6.2. 0.0. 1.0; 3.0; 5.0; 7.0; 9.0. |  |  | 7.00. 5.25. 3.50. 1.75. 0.00. 1.0; 3.0; 5.0; 7.0; 9.0. |
|  | Power Dissipated (W) |  |  | Power Dissipated (W) |

Forced Convection

|  | Heat Sink Temperature Rise Above Ambient (10W Dissipated) |  |  | Heat Sink Thermal Resistance |
| --- | --- | --- | --- | --- |
|  | Air Flow (m/s) |  |  | Air Flow (m/s) |
|  | 1.0; 2.0; 3.0; 4.1; 5.1. 15.0. 11.2. 7.5. 3.8. 0.0. 200; 400; 600; 800; 1000. |  |  | 1.0; 2.0; 3.0; 4.1; 5.1. 1.50. 1.12. 0.75. 0.38. 0.00. 200; 400; 600; 800; 1000. |
|  | Air Flow (LFM) |  |  | Air Flow (LFM) |

Building a Part Number

Full Bar Length = 8.00ft

| Base Part # | Bar Length; Finish; Length (use zeros for full or half bars). |
| --- | --- |
| 5 digit base part # | 1 Full (normally 8-ft); F Unfinished; 0 0 0 0 0. |
|  | 2 Half (small cut fee); F Unfinished; 0 0 0 0 0. |
|  | 3 Custom (cut to specified length); B Black Anodized C Gold Chromate U Unfinished* V AavSHIELD3; indicate length in inches to three decimal places; If the part is less than 10 inches, the first of these 5 digits (digit 7 of the 12 digit series) would be a zero (0).. |

| _ _ _ _ _ | _ | _ | _ _ _ _ _ |
| --- | --- | --- | --- |
| 72095 | 1 | F | 0 0 0 0 0 |

Examples:

If you wanted to simply order extrusion # 72095 in standard bar form, the part number would be:720951F00000. The unit price would be sold by the foot.

If you wanted extrusion 72095 cut to a length of 6.500" and black anodized, the part number would be:720953B06500. The unit price would be by the piece.

If you wanted extrusion 72095 cut to a length of 14.725" and a wash finish, the part number would be:720953U14725. The unit price would be by the piece.

*For unfinished extrusions with cut lengths other than half bar, the finish designation is a U.

Standard Aavid Thermalloy parts require all 12 positions to be complete.